Icas as Alarm System ”SMARTLOCK”.

Professional Alarm system for protection of personal items such as ”Bikes,Outboard engines,Garden Tools,barbecues,sateliteantennas etc.. Smartlock covers most of the need for protection against teft in a private home.

- ”Intelligent” Uses a 7m steelwire having elctronic protection against sabotage.

- Protected against tampering

- Can be connected to a sentral intruder system

- Easy installation and use

- Weather protected!

Technical spec.:

Default user code:		0000

Tamper protected  3times wrong code!	. Must be Reset thereafter.

Delays when disconnect:		20 sec.

Battery operation when activated:	Approx.9months without Alarms.

Ative Sounder time when in Alarm:	2 min. Then automatic canselled

Sounder :				90db in 1m radius.

Output to external equipment:	Open collector

Backup battery:			12VDC/1,2 Ah.

Re-charging:			12-24 hours.

Lenght of Alarm wire:		7m, can be extended to10 m.

Wire:				Steel armouring of multiple cable,

Covered in UV protected plastic hose.

Powersupply/Battery charger:	12VDC, 500mA/6VA.

Weight:				2,6 kg.

Size:				d=24,5cm  , h=8,5 cm